#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#include <DHT.h>

#include <SoftwareSerial.h>

// --------------------

// Pin Definitions

// --------------------

#define MQ5_PIN A0         // Gas leakage sensor

#define MQ2_PIN A1         // Smoke detection sensor

#define MQ135_PIN A2       // Air quality sensor

#define DHTPIN 2           // DHT11 sensor data pin

#define DHTTYPE DHT11

#define BUZZER_PIN 3       // Buzzer (alarm)

#define SERVO_PIN 5        // Servo motor

#define EXHAUST_FAN_PIN 6   // Exhaust fan

#define RELAY_PIN 7        // Relay to cut LED supply

// GSM module using SoftwareSerial on pins 8 and 9

#define GSM_RX_PIN 8  // Connect GSM module's TX to Arduino's RX on pin 8

#define GSM_TX_PIN 9  // Connect GSM module's RX to Arduino's TX on pin 9

// --------------------

// Global Objects and Variables

// --------------------

LiquidCrystal_I2C lcd(0x27, 16, 2); // Adjust the I2C address if needed

Servo myServo;

DHT dht(DHTPIN, DHTTYPE);

SoftwareSerial gsmSerial(GSM_RX_PIN, GSM_TX_PIN);

unsigned long lastSmsTime = 0;

const unsigned long smsInterval = 300000; // 5 minutes in milliseconds

// --------------------

// Function to Send SMS

// --------------------

void sendSMSAlert(float temp, int smoke, int airQuality, bool alarm) {

String message;

if (alarm) {

message = "ALERT! Gas leak or smoke detected!\n";

} else {

message = "Status Update:\n";

}

message += "Temp: " + String(temp, 1) + " C\n";

message += "Smoke: " + String(smoke) + "\n";

message += "Air Quality: " + String(airQuality);

// For debugging:

Serial.println("Sending SMS:");

Serial.println(message);

// Set GSM module to text mode

gsmSerial.println("AT+CMGF=1");

delay(1000);

// Replace with your recipient phone number (include country code)

gsmSerial.print("AT+CMGS=\"+1234567890\"\r");

delay(1000);

// Send the SMS text

gsmSerial.print(message);

delay(1000);

// Send Ctrl+Z to signify SMS end

gsmSerial.write(26);

delay(5000);

}

// --------------------

// Setup

// --------------------

void setup() {

Serial.begin(9600);

gsmSerial.begin(9600);

// Initialize LCD and display a start-up message

lcd.init();

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("System Starting");

delay(2000);

lcd.clear();

// Initialize the DHT11 sensor

dht.begin();

// Attach servo motor and set its initial position

myServo.attach(SERVO_PIN);

myServo.write(0);

// Set actuator pins as OUTPUT

pinMode(BUZZER_PIN, OUTPUT);

pinMode(EXHAUST_FAN_PIN, OUTPUT);

pinMode(RELAY_PIN, OUTPUT);

// Ensure actuators are off initially

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(EXHAUST_FAN_PIN, LOW);

digitalWrite(RELAY_PIN, LOW);

Serial.println("System Initialized...");

}

// --------------------

// Main Loop

// --------------------

void loop() {

// --------------------

// Read Sensors

// --------------------

int mq5Value = analogRead(MQ5_PIN);       // Gas leakage sensor reading

int mq2Value = analogRead(MQ2_PIN);         // Smoke sensor reading

int mq135Value = analogRead(MQ135_PIN);       // Air quality sensor reading

float temperature = dht.readTemperature();

// (Optionally read humidity) float humidity = dht.readHumidity();

// For display, assign:

int smokeLevel = mq2Value;    // Use MQ2 reading for smoke

int airQuality = mq135Value;   // Use MQ135 reading for AQ

int gasLeakValue = mq5Value;   // Use MQ5 reading for gas leak

// Debug prints to Serial Monitor

Serial.print("Gas (MQ5): ");

Serial.print(gasLeakValue);

Serial.print(" | Smoke (MQ2): ");

Serial.print(smokeLevel);

Serial.print(" | Air Quality (MQ135): ");

Serial.print(airQuality);

Serial.print(" | Temp: ");

Serial.println(temperature);

// --------------------

// Update LCD Display

// --------------------

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature, 1);

lcd.print("C ");

lcd.print("S:");

lcd.print(smokeLevel);

lcd.setCursor(0, 1);

lcd.print("AQ:");

lcd.print(airQuality);

lcd.print("      ");  // Clear any leftover characters

// --------------------

// Check for Gas Leak/Smoke and Activate Alarm

// --------------------

// Sample threshold values (calibrate these as needed)

const int gasThreshold = 400;

const int smokeThreshold = 400;

bool alarmTriggered = false;

if (gasLeakValue > gasThreshold || smokeLevel > smokeThreshold) {

alarmTriggered = true;

// Activate alarms and devices

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(EXHAUST_FAN_PIN, HIGH);

digitalWrite(RELAY_PIN, HIGH);

myServo.write(90);  // For example, a servo movement as a warning indicator

// Change LCD display to show alert

lcd.setCursor(0, 1);

lcd.print("!! ALERT !!    ");

// Send an immediate SMS alert (if necessary)

sendSMSAlert(temperature, smokeLevel, airQuality, true);

} else {

// No alarm: make sure all alert devices are deactivated

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(EXHAUST_FAN_PIN, LOW);

digitalWrite(RELAY_PIN, LOW);

myServo.write(0);

}

// --------------------

// Periodic SMS Update Every 5 Minutes

// --------------------

if (millis() - lastSmsTime >= smsInterval) {

lastSmsTime = millis();

sendSMSAlert(temperature, smokeLevel, airQuality, false);

}

// A small delay before the next reading

delay(2000);

}

Other code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#include <DHT.h>

#include <SoftwareSerial.h>

// --------------------

// Pin Definitions

// --------------------

#define MQ5_PIN A0         // Gas leakage sensor

#define MQ2_PIN A1         // Smoke detection sensor

#define MQ135_PIN A2       // Air quality sensor

#define DHTPIN 2           // DHT11 sensor data pin

#define DHTTYPE DHT11

#define BUZZER_PIN 3       // Buzzer (alarm)

#define SERVO_PIN 5        // Servo motor

#define EXHAUST_FAN_PIN 6  // Exhaust fan

#define RELAY_PIN 7        // Relay to cut LED supply

// GSM module using SoftwareSerial on pins 0 and 1

#define GSM_RX_PIN 0  // Connect GSM module's TX to Arduino's RX on pin 0

#define GSM_TX_PIN 1  // Connect GSM module's RX to Arduino's TX on pin 1

// --------------------

// Global Objects and Variables

// --------------------

LiquidCrystal_I2C lcd(0x27, 16, 2); // Adjust the I2C address if needed

Servo myServo;

DHT dht(DHTPIN, DHTTYPE);

SoftwareSerial gsmSerial(GSM_RX_PIN, GSM_TX_PIN);

unsigned long lastSmsTime = 0;

const unsigned long smsInterval = 300000; // 5 minutes in milliseconds

// --------------------

// Function to Send SMS

// --------------------

void sendSMSAlert(float temp, int smoke, int airQuality, bool alarm) {

String message;

if (alarm) {

message = "ALERT! Gas leak or smoke detected!\n";

} else {

message = "Status Update:\n";

}

message += "Temp: " + String(temp, 1) + " C\n";

message += "Smoke: " + String(smoke) + "\n";

message += "Air Quality: " + String(airQuality);

// For debugging:

Serial.println("Sending SMS:");

Serial.println(message);

// Set GSM module to text mode

gsmSerial.println("AT+CMGF=1");

delay(1000);

// Replace with your recipient phone number (include country code)

gsmSerial.print("AT+CMGS=\"+1234567890\"\r");

delay(1000);

// Send the SMS text

gsmSerial.print(message);

delay(1000);

// Send Ctrl+Z to signify SMS end

gsmSerial.write(26);

delay(5000);

}

// --------------------

// Setup

// --------------------

void setup() {

Serial.begin(9600);

gsmSerial.begin(9600);

// Initialize LCD and display a start-up message

lcd.init();

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("System Starting");

delay(2000);

lcd.clear();

// Initialize the DHT11 sensor

dht.begin();

// Attach servo motor and set its initial position

myServo.attach(SERVO_PIN);

myServo.write(0);

// Set actuator pins as OUTPUT

pinMode(BUZZER_PIN, OUTPUT);

pinMode(EXHAUST_FAN_PIN, OUTPUT);

pinMode(RELAY_PIN, OUTPUT);

// Ensure actuators are off initially

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(EXHAUST_FAN_PIN, LOW);

digitalWrite(RELAY_PIN, LOW);

Serial.println("System Initialized...");

}

// --------------------

// Main Loop

// --------------------

void loop() {

// --------------------

// Read Sensors

// --------------------

int mq5Value = analogRead(MQ5_PIN);       // Gas leakage sensor reading

int mq2Value = analogRead(MQ2_PIN);      // Smoke sensor reading

int mq135Value = analogRead(MQ135_PIN);  // Air quality sensor reading

float temperature = dht.readTemperature();

// (Optionally read humidity) float humidity = dht.readHumidity();

// For display, assign:

int smokeLevel = mq2Value;    // Use MQ2 reading for smoke

int airQuality = mq135Value; // Use MQ135 reading for AQ

int gasLeakValue = mq5Value; // Use MQ5 reading for gas leak

// Debug prints to Serial Monitor

Serial.print("Gas (MQ5): ");

Serial.print(gasLeakValue);

Serial.print(" | Smoke (MQ2): ");

Serial.print(smokeLevel);

Serial.print(" | Air Quality (MQ135): ");

Serial.print(airQuality);

Serial.print(" | Temp: ");

Serial.println(temperature);

// --------------------

// Update LCD Display

// --------------------

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature, 1);

lcd.print("C ");

lcd.print("S:");

lcd.print(smokeLevel);

lcd.setCursor(0, 1);

lcd.print("AQ:");

lcd.print(airQuality);

lcd.print("      ");  // Clear any leftover characters

// --------------------

// Check for Gas Leak/Smoke and Activate Alarm

// --------------------

// Sample threshold values (calibrate these as needed)

const int gasThreshold = 400;

const int smokeThreshold = 400;

bool alarmTriggered = false;

if (gasLeakValue > gasThreshold || smokeLevel > smokeThreshold) {

alarmTriggered = true;

// Activate alarms and devices

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(EXHAUST_FAN_PIN, HIGH);

digitalWrite(RELAY_PIN, HIGH);

myServo.write(90);  // For example, a servo movement as a warning indicator

// Change LCD display to show alert

lcd.setCursor(0, 1);

lcd.print("!! ALERT !!    ");

// Send an immediate SMS alert (if necessary)

sendSMSAlert(temperature, smokeLevel, airQuality, true);

} else {

// No alarm: make sure all alert devices are deactivated

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(EXHAUST_FAN_PIN, LOW);

digitalWrite(RELAY_PIN, LOW);

myServo.write(0);

}

// --------------------

// Periodic SMS Update Every 5 Minutes

// --------------------

if (millis() - lastSmsTime >= smsInterval) {

lastSmsTime = millis();

sendSMSAlert(temperature, smokeLevel, airQuality, false);

}

// A small delay before the next reading

delay(2000);

}